# 0 - Preliminary

What to do before starting the assembly

- Using the parts list, make sure you have all the components.

- The parts list Part letters will be referenced throughout these instructions.

- You will need to supply the following:

  - Soldering iron

  - hot glue gun

  - Philips head screwdriver

  - 9-volt battery

- Mentally prepare for this to take about 2-3 hours.

# 1 - Printing the STL’s

STL’s can be found on the project info page, which is linked at the QR code under the parts list.

- The case bottom needs supports, nothing else needs supports.

- I would recommend printing the “Case Parts” group in one color, the “Cap Parts” group in a different color, and the “Button Springs” group in the same color as the “Case Parts” groups.

- I would also recommend printing the button springs on their own build plate with nothing else on it. They are the hardest part to print.

- While the STL’s are printing, move on to Soldering!

# 2 - Soldering

Note: although where the parts go into the PCB is mostly self-explanatory, the order you solder them on does matter.

- Solder 4x G (Push Button) on top of V (PCB)

- Solder U (Arduino Mega Pro) pins onto U. The tall end of these pins goes into the bottom of U and out the top. Place the pins into V (PCB) while you solder so there are no alignment issues. Please refer to the tutorial video if you are unsure of how to do this step.

- Solder U (Arduino Mega Pro) on bottom of V (PCB)

- Solder 2x J (Potentiometer) on top of V (PCB)

- Solder T (Female Header) on top of V (PCB)

- Solder I (Power Switch) on top of V (PCB) Make sure it is in the correct orientation!

- Solder R (Gyro) pins on R. The short pins go into the bottom of R and out the top.

- Solder R (Gyro) on bottom of V (PCB)

- Solder 2x O (Joystick) on top of V (PCB)

- Solder Q (NRF24L01) on bottom of V (PCB)

The order of everything else does not matter.

- Solder 4x F (Toggles) on top of V (PCB)

- Solder 2x N (JST Connector Female) on top of V (PCB) on “Bumper 1” & “Bumper 3”

- Solder 2x N (JST Connector Female) on bottom of V (PCB) on “Bumper 2” & “Bumper 4”

- Solder N (JST Connector Female) on bottom of V (PCB) on “Battery1” Make sure it is in the correct orientation!

- Solder P (Rotary Encoder) on top of V (PCB)

- Solder 2x L (10uf Capacitor) on bottom of V (PCB). Make sure they are in the correct orientation!

- Solder 2x K (10k Resistor) on bottom of V (PCB)

- Solder 2x K (10k Resistor) on top of V (PCB)

- Solder 4x M (JST Connector Male) on the H (Bumper Switch) pins “C” and “NO”.

  - The wires should be cut to a length of about 2 inches before soldering.

  - These are the 2 pins on the left in the Parts list picture of H

  - Which wire gets soldered to which H pin does not matter if you are soldering to the correct 2 H pins.

# 3 - Assembly

How to put it together! You’ll need a screw driver, hot glue gun, and a 9 volt battery.

- Insert 10x D (M3 Heated Inserts) There are 6 in the case bottom, and 4 in the case top.

- Attach 2x E (Battery Contact Spring) using 2x A (M2 Battery Tab Screw 6mm). Leave a small gap, do not tighten down yet.

- Find the M (JST Connector Male) that you did not connect to a bumper. Insert it’s wires under the 2x E (Battery Contact Spring). The right is positive, and the left is negative. Tighten the 2x E. Do not overtighten.

- Attach the 4x Bumper Caps to the 4x H (Bumper Switch). Use a dab of hot glue before attaching so they stay on.

- Find the 2x H (Bumper Switch) with the M (JST Connector Male) soldered to them that were not cut short. Attach them to the case bottom using 4x B (M2 Bumper Screw 10mm)

  - The side that gets pressed in should face inward.

  - Make sure they can be pressed without getting stuck.

  - Do not overtighten.

- Plug in the 3x M (JST Connector Male) wires into the associated 3x N (JST Connector Female) that are on the PCB.

- Attach V (PCB) to the case bottom using 2x C (M3 Screw 10mm). Make sure the 3x M (JST Connector Male) wires don’t get pinched.

- Insert the 2x screen spacer 3d prints onto V (PCB) and plug in S (Screen)

- Repeat Step 5 on the case top with the remaining 2x H (Bumper Switch).

- Plug in the 2x M (JST Connector Male) wires into V (PCB) and attach the top case to the bottom case. Make sure the wires don’t get pinched

- Attach the 3d printed caps.

  - 2x J (Potentiometer) caps

  - 2x O (Joystick) caps. These may need hot glue.

  - 1x P (Rotary Encoder) cap.

- Attach 4x button springs to the top case. You may need to use a tool to properly compress the connectors into the top case. They should at least be flush.

- Attach the 4x button caps to the button springs. A to D, top to bottom.

- Screw in the 2 side handles using 8x C (M3 Screw 10mm). Do not overtighten.

- Screw on Q (NRF24L0) Antenna

- Insert 9 Volt battery (Not included). ‘+’ on the right, ‘-‘ on the left.

- Attach 3d printed battery door.

- Power on!